| Devices | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | - | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| PC1 | - | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |
| PC2 | - | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 |
| PC3 | - | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 |
| PC4 | - | 192.168.3.2 | 255.255.255.0 | 192.168.3.1 |
| PC5 | - | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 |
| Switch0 | VLAN1 | 192.168.1.5 | 255.255.255.0 |  |
| Switch1 | VLAN1 | 192.168.2.5 | 255.255.255.0 |  |
| Switch2 | VLAN1 | 192.168.3.5 | 255.255.255.0 |  |
| Router0 | Int g0/0 | 192.168.4.1 | 255.255.255.0 |  |
|  | Int g0/1 | 192.168.1.1 | 255.255.255.0 |  |
|  | Int g0/2 | 192.168.2.1 | 255.255.255.0 |  |
| Router1 | Int g0/0 | 192.168.4.2 | 255.255.255.0 |  |
|  | Int g0/1 | 192.168.3.1 | 255.255.255.0 |  |
|  | Int g0/2 | - | - | - |

02.

04.

a.

Switch>enable

Switch#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname S0

b.

S0#show running-config

Building configuration...

Current configuration : 1076 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname S0

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

!

!

!

line con 0

!

line vty 0 4

login

line vty 5 15

login

!

!

!

!

end

S0#

c.

S0#conf t

Enter configuration commands, one per line. End with CNTL/Z.

S0(config)#line vty 0 15

S0(config-line)#password cisco

S0(config-line)#login

d.

S0(config-line)#exit

S0(config)#

S0(config)#line con 0

S0(config-line)#password cisco

S0(config-line)#login

e.

S0(config)#exit

S0#

%SYS-5-CONFIG_I: Configured from console by console

S0#

S0#show start

S0#show startup-config

startup-config is not present

f.

S0#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

S0#

g.

S0#show flash

Directory of flash:/

1 -rw- 4670455 <no date> 2960-lanbasek9-mz.150-2.SE4.bin

2 -rw- 1131 <no date> config.text

64016384 bytes total (59344798 bytes free)

S0#

S0#show startup-config

Using 1131 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname S0

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

!

!

!

line con 0

password cisco

login

!

line vty 0 4

password cisco

login

line vty 5 15

password cisco

login

!

!

!

!

end

S0#

05.

Router0

Router(config)#ip route 192.168.3.0 255.255.255.0 192.168.4.2

Show ip route output

Router#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

192.168.1.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.1.0/24 is directly connected, GigabitEthernet0/1

L 192.168.1.1/32 is directly connected, GigabitEthernet0/1

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.2.0/24 is directly connected, GigabitEthernet0/2

L 192.168.2.1/32 is directly connected, GigabitEthernet0/2

S 192.168.3.0/24 [1/0] via 192.168.4.2

192.168.4.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.4.0/24 is directly connected, GigabitEthernet0/0

L 192.168.4.1/32 is directly connected, GigabitEthernet0/0

Router#

Router1

Router(config)#ip route 192.168.1.0 255.255.255.0 192.168.4.1

Show ip route output

Router#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

S 192.168.1.0/24 [1/0] via 192.168.4.1

S 192.168.2.0/24 [1/0] via 192.168.4.1

192.168.3.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.3.0/24 is directly connected, GigabitEthernet0/1

L 192.168.3.1/32 is directly connected, GigabitEthernet0/1

192.168.4.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.4.0/24 is directly connected, GigabitEthernet0/0

L 192.168.4.2/32 is directly connected, GigabitEthernet0/0

Router#