- Router

A router connects different networks and decides how data should travel between them.

It connects various networks simultaneously.

Switch

A switch connects devices within a single network.

It connects various devices simultaneously.

- Copper Straight-Through cables

06.

Router>enable

Router#conf

Router#configure t

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface g

Router(config)#interface gigabitEthernet 0/0

Router(config-if)#ip a

Router(config-if)#ip address

Router(config-if)#ip address 200.165.10.10 255.255.255.0

Router(config-if)#no shut

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

Router(config-if)#exit

Router(config)#inter

Router(config)#interface G

Router(config)#interface GigabitEthernet 0/1

Router(config-if)#ip address

Router(config-if)#ip address 200.165.20.10

% Incomplete command.

Router(config-if)#ip address

Router(config-if)#ip address 200.165.20.10 255.255.255.0

Router(config-if)#no shut

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up

Router(config-if)#exit

Router(config)#interface g0/2

Router(config-if)#ip addre

Router(config-if)#ip address 200.165.30.10 255.255.255.0

Router(config-if)#no dh

Router(config-if)#no dh

^

% Invalid input detected at '^' marker.

Router(config-if)#no shu

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/2, changed state to up

Router(config-if)#exit

Router(config)#copy ru

Router(config)#^Z

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#copy ru

Router#copy running-config sta

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

07.

10.

Ping from PC0 to Laptop2

Ping from PC3 to PC1

Ping from Laptop0 to Laptop1

11.

a.

Switch>enab

Switch>enable

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#host

Switch(config)#hostname S0

S0(config)#

b.

S0(config)#^Z

S0#

%SYS-5-CONFIG_I: Configured from console by console

S0#show runn

S0#show running-config

Building configuration...

Current configuration : 1076 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname S0

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

!

!

!

line con 0

!

line vty 0 4

login

line vty 5 15

login

!

!

!

!

end

c.

S0#

S0#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

S0(config)#line vty 0 15

S0(config-line)#passw

S0(config-line)#password cisco

S0(config-line)#login

S0(config-line)#exit

d.

S0(config)#line con 0

S0(config-line)#pass

S0(config-line)#password cisco

S0(config-line)#login

S0(config-line)#ex

S0(config-line)#exit

e.

S0(config)#exit

S0#

%SYS-5-CONFIG_I: Configured from console by console

S0#show star

S0#show startup-config

startup-config is not present

f.

S0#copy running-config st

S0#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

g.

S0#show flash:

Directory of flash:/

1  -rw-     4670455          <no date>  2960-lanbasek9-mz.150-2.SE4.bin

2  -rw-        1131          <no date>  config.text

64016384 bytes total (59344798 bytes free)

12.